# COMBINED CYCLE POWER PLANT by ANSALDO (280 +160 MW)

The CCPP is composed by:

N.1 ANSALDO GAS TURBINE type V94.3A4 with electrical generator air cooler type WY23Z-109 m Rated power 280 MW 50hz and all relative auxiliary systems.

  - ANSALDO STEAM TURBINE with electrical generator air cooled WX21Z-092 and relative auxiliary systems. Rated power 160 MW.

The Plant is completed in all parts and relative auxiliary.

The plant is stocked in logistic cover plant, packaged and ready to ship. Was made in Italy YOM 2021 Never Used — NEW 0 Hours

PRICE COMPLETE PLANT EX WORKS  65.000.0000 Euro

For all any other info in we sent you complete documentation in file.zip (595 pages)

    - There are about 1000 V engines operating worldwide. This total includes both E- and F- class models operating at 50 and 60 Hz, providing plenty of experiences to share across national borders.

    - Two OEMs manufacture V engines, Siemens and Ansaldo Energia.  The engine designations for Siemens machines are SGT 5-4000F (V94.3A, 50 Hz), SGT 52000E (V94.2, 50 Hz), SGT6-4000F (V84.3A, 60 Hz), and SGT6-2000E

(V84.2, 60 Hz). For Ansaldo engines, the designations are AE94.3A and AE94.2—50 Hz only. Roughly 80% of all V machines in service were made by Siemens, the balance by Ansaldo under a Siemens license.

# Specifications

## SCOPE OF SUPPLY

Manufacturer	Ansaldo Energia

Model	AE94.3A

Year	2021

Condition	New

Wattage	288 +160

Hours	Zero

Frequency	50 Hz

Fuel Type	Natural Gas

Voltage	10500

Figure 2:	Gas turbine longitudinal section

9

4

1 Front beanno pedestalwtth compressor bear no

2  Compressor vme earner 2

3   Burrer

- Turbine outer casing

- Compressor vme canter 1

- CombUst ion chamber outer casing

7    Annular combustiOn chamber

- Turbine vane CCJ"rier

- Turbine beanng pedestalwih emaus t castng

## AUXILIRY  SYSTEMS ( included)

- GAS TURBINE AUXILIARIES

  - LUBE AND LIFTING OIL

  - HYDRAULIC OIL

  - GAS TURBINE ON BOARD DRAINAGE

  - BLOW OFF

  - TURBINE COOLING

  - GAS TURBINE MONITORING

  - COMBUSTION CHAMBER MONITORING

  - FUEL GAS

  - FUEL OIL

  - HIGH PRESSURE PURGING

- GAS TURBINE ACCESSORIES

  - COMPRESSOR WASHING

  - AIR INTAKE

  - EXHAUST GAS DUCT

## STEAM TURBINE DESIGN FOR THE ADVANCED COMBINED CYCLE SCOPE OF SUPPLY

STEAM TURBINES RT20C-33 TYPE

Efficient, flexible and reliable machines Steam turbines are the foundations on which Ansaldo was built: the Company was established in 1853 to support the development of the then-flourishing steam locomotive industry.

Ansaldo Energia long-standing experience on steam covers the design, manufacturing and installation of steam engines: the overall unit amount is about 700 worldwide with a total capacity of nearly 100 GW. Today’s Ansaldo Energia steam turbine portfolio has been developed for application in most power generation technologies, from traditional fossil combustion to renewable energy:

    - Steam Power and Combined Cycle plants

    - District heating, cogeneration, desalinization and solar power facilities, as well as nuclear plants

Current production includes:

- Large ratings for reheat applications, respecting the highest steam parameters for supercritical and ultra-supercritical conditions

- Compact modules for smaller ratings

- Single and two-cylinders models for non-reheat thermal cycles

  - Geothermal steam turbines, based on impulse design Each steam turbine model is a combination of pre-engineered, well-proven modules of different sizes, providing a broad range of power ratings and applications in both 50 and 60 Hz markets.

Modular design reduces both technical development and manufacturing time.

All models are assembled in our workshop – except the large low-pressure sections, which are most conveniently field-installed

REHEAT Efficient and flexible

Reheat steam turbines increase efficiency by returning partially-expanded steam to the steam generator for reheating before feeding it back to the turbine. The Ansaldo Energia reheat steam turbines portfolio includes machines from two to five cylinders, capable of covering a wide range of steam cycles, site conditions and operating modes.

For small ratings, the design concept is based on two cylinders with singleflow exhaust. For higher ratings, the design is based on separate high-pressure and intermediate- pressure sections with one (or more) double-flow low-pressure sections.

This configuration is suitable for Combined Cycles with advanced F- and H- class gas turbines as well as fossil power plants with the highest steam parameters for supercritical/ultra-supercritical    conditions.

Application	Large Reheat CC or Fossil-fired Steam Cycles,

Steam parameters	Up to 280 bar Up to 600°C / 620°C

Configuration	Three or more cylinders with double f low LP section(s)

- Main steam system

- Gland steam system

  - Sealing steam systems MAW10/15

  - Leak-off system MAW30

  - Operating guidelines

- Lube oil and hydraulic system.

  - Lube oil system

  - Jacking oil system.

  - Turning gear system

  - Hydraulic actuation system.

  - Lube oil purifier system

- Control and safety systems

  - Turbine safety system S90

  - Turbine control system

  - Hydraulic components of the safety and control system

  - Test system

- Reverse power protection

- Vacuum breaking system

- Ventilation system.

- Water injection into the LP exhaust

- Drain system

  - General.

  - Classification of the drains

- Turbine Supervisory Instrumentation system (TSI

  - Rotor relative vibrations

  - Differential expansion channel.

  - Axial position (thrust bearing wear)

  - Absolute case expansion

  - Phase angle channel

- Generator outline

The generator is an air-cooled, 2-pole machine with cylindrical rotor, ventilated in closed circuit using air-to-water heat exchangers located in the foundation in case of WX configuration or in the lower part of the stator frame in case of WY configuration. In last case a small pit completes the

air-cooling path.

The WY arrangement (Figure 1) is adopted when the generator is coupled to gas turbine and steam turbine with lateral or axial steam discharge, while WX configuration (Figure 2) is generally adopted when coupled to steam turbine with vertical discharge from below.

## Main features

- Two pole turbogenerator with cylindrical rotor

- Rotor directly coupled to the turbine

- Rotor equipped with damper winding

- Closed circuit cooling with air-to-water heat exchangers 4 incorporated into the housing (WY, Figure 3) or

4 in the foundation (WX)

- Self ventilating air cooling system for stator and rotor

- Generator shipped completely assembled (WY if weights and dimensions match the transportation limitation) or disassembled (WX)

- Static excitation system

- Stator winding with single bar insulation system

- IEC standard reference

- Class F insulation system for stator and rotor

- Shaft grounding device and non-drive end bearing insulation to prevent damages in the bearings.

## Description

The following description is mainly related to the WY solution but most of it is also valid for WX configuration.

The 2-pole 3 phase synchronous generator is air cooled in closed circuit. A welded housing that is

split horizontally to enable access for maintenance and repair work encloses it. The rotor turns in two radial friction bearings that are located outside the generator housing and are fully separate from the housing. This feature makes maintenance significantly easier. The axial position of the shaft is maintained by the thrust bearing on the turbine shaft. The non-driving end bearing has double insulation. In addition, the shaft is grounded to protect the bearings and their supply systems from damage due to eddy currents in the shaft. Lube oil and jacking oil are supplied by  the turbine oil systems.

The hot air, warmed in the machine, is cooled by heat exchangers arranged cross-wise in the bottom section of the generator housing (WY) or in the foundation (WX). The heat is carried off in cooling water that is cooled in a remote cooling system. The cooling system for the generator cooling air is described in more detail in the Chapter, "Generator cooling system".

The rotor is equipped with a damper winding to allow operation with asymmetric load in accordance to IEC and IEEE Standards.

The six terminals of the stator are connected on the topside of the generator. They are built into

the top half of the housing (WY) or go out from the bottom (WX), at the generator non-driving end.

The brush gear is placed at the non-driving end on the generator frame (WY) or on the concrete (WX). The design allows supervision and replacement of the brushes while the generator is running under load.

## Generator Cooling System

4.  Main Features

- Air-cooling in a closed circuit, with temperature maintained even with built-in water-to-air heat exchangers

- Two axial fans on the shaft to circulate the cooling air

- Two parallel cooling systems arranged parallel to the main axis of the generator.

## Stator Cooling

To support the cooling, the stator housing is divided into six chambers on each end. One portion of the air blown into the generator by the axial fans (1) goes directly into the air gap between the

rotor and the stator (2). Mixed with the air from the ends of the rotor coil, it leaves this zone through radial cooling slots in the stator and, passing through the outer zones of the stator housing, reaches the coolers (3) installed in the bottom half of the housing. From there, the airflows back to the fans. The rest of the air passes first, for cooling purposes, the ends of the

stator windings (4), and is then directed through axial pipes into the second (5), forth (6) and sixth

(7) chamber inside the housing. There it flows inward into the air gap, through the cooling slots in

the stator, where it mixes with the cooling air in the air gap. Then the air flows in the counter direction through the adjoining chambers and passes through the radial openings in the stator to reach the outer chamber of the housing. This then directs the air to the heat exchanger.

The brush gear cooling system is separated from the generator cooling system. The slip ring fan sucks in cooling air from the ambient through air filters on one side and exhausts on the opposite side or above. The make-up filters are provided with silencers.

## Rotor Cooling

The cooling air flows between the retaining rings and the shaft to enter the end windings part. From there, the air flows into the hollow conductors, where it divides up. One portion flows through the hollow conductors in the slot as far as the middle of the rotor, where it comes out into the air gap through radial holes in the hollow conductors and the slot wedges. The other portion flows through the hollow conductors in the end winding as far as the middle of the rotor, where  the air leaves the hollow conductors. Ultimately the air flows out into the air gap through slots at the end of the rotor body. In addition, air enters into subslots and cools the rotor winding in the middle of the rotor (area of chambers 4,5 and 6).

Because the air outlet is located on a radius larger than that of the air inlet, the rotor itself forces the cooling airflow. That means the rotor is self-ventilated.

## Heat exchangers

The air warmed in the generator is cooled off in surface heat exchangers, with water as the secondary cooling medium. The generator cooler is made up of four heat exchangers installed across the bottom of the generator and with water and air connections in parallel. In case of one cooler out of service the generator operates at reduced load as per standard application.

The simple design of the heat exchangers makes them easy to clean with a brush.

Based on the water type and quality, suitable materials are selected for tubes, tube plates, fins and water chambers.

## Instrumentation ( included)

The instrumentation is built directly into the generator and used mainly to monitor temperatures and its general operation.

- For both WY and WX configuration, the instrumentation includes:

12 RTD double Pt100 in the stator winding between top and bottom bars into the slots 4 RTD double Pt100 in the stator core

2 Double thermo-elements in the bearing metal in each bearing shell (one each bearing) 2 sight glasses in the oil return from each bearing (one each bearing)

2 dial thermometers in the bearing oil drain (one each bearing) 2 pressure indicator, dial type, at the oil inlet (one each bearing)

2 Vibrations detectors on the bearings pedestals (one each bearing)

2 Shaft vibrations detectors each bearing, mounted at 90° one each other 1 RTD double Pt100 on the hot air at the slip rings housing outlet

1 Connection for insulation resistance measurement on the non-driving end bearing

Moreover, depending on the mounting arrangement the following instrumentation is also foreseen:

- For WY generator type:

4 RTD double Pt100 for cold air at the coolers outlet (one each cooler) 4 RTD double Pt100 for hot air at the coolers inlet (one each cooler)

4 RTD double Pt100 at fans upstream (two each fan) 4 Liquid leakage detectors

2  Differential pressure gauges across the slip-rings housing air filters

- For WX generator type:

1 RTD double Pt100 for each column for cold air at the coolers outlet 1 RTD double Pt100 for each column for hot air at the coolers inlet

4 RTD double Pt100 at fans upstream (two each fan) 2 RTD double Pt100 at core outlet

1 Liquid leakage detector

1 Differential pressure gauge across the slip rings housing air filte

## CONTROL SYSTEMS

This document describes the characteristics of the Gas Turbine Control Monitoring and Protection System (GTCMPS) supplied for the Ansaldo Gas Turbine Dual Fuel V94.3A4 in configuration multi shaft.

GTCMPS is based on the Symphony Harmony Control System from ABB. The Symphony Harmony is a microprocessor-based digital distributed control system.

The gas turbine control, monitoring and protection system are standard HW and SW package that has been developed specifically to perform all control, protection and supervisory functions of the Ansaldo V94.3A4 Gas Turbine Generator Set .

The GTCMPS has the HSI for local operation, consisting of two Operation Stations (OS) and one Engineering Station (EWS),

The GTCMPS cabinets and the TSI will be installed in a local control room (LCR) foreseen

near to the gas turbine machine, while an other operator interfaces (1 OS) will be installed in the central control room (CCR).

The cubicles of the GTCMPS are connected between them through high-speed control bus (C-NET) for data exchanging in order to realise a common redundant ring and to be time- synchronised together.

Hardwired connections are also provided for safety and protection exchange of signals.

This redundant hardware and communication media assure high system reliability and availability. The back-up line automatically takes over in the event of a fault in the primary one.

Efficiency is guaranteed by use of exception reporting technique.

Erroneous data transmission is avoided by means of extensive use of error checking and message acknowledgement.

The data highway uses coaxial cable as transmission media

All external systems are controlled by the plant DCS.

The plant DCS can be built with the same technology (Symphony Harmony) OR with other

ABB technology OR by other manufacturer.

In this plant the GTCMPS will be connected to the DCS (made with ABB technology not Symphony Harmony type) by means of redundant Ethernet TCI/IP from the switches and by means HW signals. All Ansaldo components (printers, OS, EWS) will be connected between them by means of the redundant Ethernet TCI/IP. (See figure at point 3) Anywhere, hardwired trip are connected between the DCS, ESD and the GTCMPS.

The GTCMPS is designed for full automatic operation of the ANSALDO V94.3A4 Gas turbine generator set and perform the main following functions:

- Automatic start up, Operation and Shutdown

- Drive control

- Fuel control

- Monitoring

- Gas turbine protection

The GTCMPS will include the control, protection regulation and supervisory of the following systems:

- Lube oil

- Combustion chamber

- Air Intake

- Hydraulic oil

- Blow off

- Pneumatic actuators for blow off

- Hydraulic actuators for fuel gas/fuel oil

- Pneumatic system

- Exhaust system

- Turbine cooling

- Fuel gas

- Fuel oil

- Ignition

- Drainage

- Gas turbine (bearings, compressor and turbine pressure and temperatures)

- Excitation

- Static Frequency Converter

- Generator interlock (the supervisory is excluded)

- Interface with DCS